Hoelscher Farms – Tract 3

FIELD NOTES for a 10.03 acre tract of land out of the southwest quarter of Section 12, Block D5, EL&RR Ry. Co. Survey, Abstract 735, Lubbock County, Texas, more particularly described as follows:

BEGINNING in the south line of said Section 12 in County Road 5700 which bears N. 88º 18’ 20” W. a distance of 878.13 feet from a 2-3/4” pipe found at the southeast corner of said southwest quarter for the southeast corner of this tract.

THENCE N. 88º 18’ 20” W., along said south line and said County Road 5700, a distance of 325.50 feet to the southwest corner of this tract.

THENCE N. 01º 57’ 59” E., at 30.00 feet pass a 1/2" iron rod set with a cap inscribed “OJD-ENG” (hereafter referred to as an OJD cap) in the north margin of said County Road 5700, a total distance of 1342.65 feet to an OJD cap set for the northwest corner of this tract.

THENCE S. 88º 02’ 01” E. a distance of 325.50 feet to an OJD cap set for the northeast corner of this tract.

THENCE S. 01º 57’ 59” W., at 1311.11 feet pass an OJD cap set in the north margin of County Road 5700, a total distance of 1341.10 feet to the POINT OF BEGINNING and containing 10.03 acres of land.